Brian Lopez

CPE301 – SPRING 2018

# Design Assignment 4

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 1 | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 2. | INITIAL CODE OF TASK 1/A |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 4. | SCHEMATICS |  |  |
| 5. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 5. | SCREENSHOT OF EACH DEMO |  |  |
| 6. | VIDEO LINKS OF EACH DEMO |  |  |
| 7. | GOOGLECODE LINK OF THE DA |  |  |

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmega328P

- FTDI (only for providing power)

- DC motor, Stepper motor, and servo motor for tasks 1, 2, and 3 respectively

- Potentiometer

- Jumper wires

- INITIAL/DEVELOPED CODE OF TASK 1/A

#define F_CPU 8000000

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

int val = 0;

void ADC_init() {

ADMUX = 0;                 // read from port ADC0

ADMUX |= (1<<REFS0);       // use AVcc for reference

ADCSRA |= (1<<ADPS2) | (1<<ADPS1); // prescalar of 64

ADCSRA |= (1<<ADEN);       // enable ADC

ADCSRB = 0;                // free running mode

}

unsigned int readADC()

{

ADMUX &= ~(1<<ADLAR);              // clear the adc value

ADCSRA |= (1 << ADSC);             // start adc

while(ADCSRA & (1<<ADSC));         // wait until adc is done

return ADC;

}

int main(void)

{

ADC_init();

DDRB |= (1 << 1) | (1 << 2);   // set pins portb.1 and 2 as output

PORTB = (1 << 2);              // set B2 as high for clockwise rotation

DDRD |= (1 << 6);              // set D6 as output to control enable on driver

OCR0A = 243;

TCCR0A |= (1 << COM0A1);               // non-inverted mode

TCCR0A |= (1 << WGM02) | (1 << WGM00); // phase correct PWM

TCCR0B |= (1 << CS01);                 // prescalar of 8

while(1)

{

val = readADC() / 4.20;  // read ADC and divide by 4.2 to not exceed 243

OCR0A = val;             // which is 95% of PWM value. then update PWM compare

}

}

- TASK 2 CODE

#define F_CPU 8000000

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

volatile int step;

void ADC_init() {

ADMUX = 0;                 // read from port ADC0

ADMUX |= (1<<REFS0);       // use AVcc for reference

ADCSRA |= (1<<ADPS2) | (1<<ADPS1); // prescalar of 64

ADCSRA |= (1<<ADEN);       // enable ADC

ADCSRB = 0;                // free running mode

}

unsigned int readADC()

{

ADMUX &= ~(1<<ADLAR);              // clear the adc value

ADCSRA |= (1 << ADSC);             // start adc

while(ADCSRA & (1<<ADSC));         // wait until adc is done

return ADC;

}

ISR(TIMER0_COMPA_vect)

{

if (step == 0)           // change position of motor depending

PORTB = (1 << 1);    // on which step it is currently at

else if (step == 1)

PORTB = (1 << 2);

else if (step == 2)

PORTB = (1 << 3);

else if (step == 3)

PORTB = (1 << 4);

step++;

if (step == 4)        // increment and reset step if necessary

step = 0;

_delay_us(500);           // small delay to let motor move

OCR0A = 255  - readADC() / 4.02;      // change Compare value to ADC value

// divide 4.02 so that max value is 254.

TIFR0 |= (1 << OCF0A);       // reset interrupt flag

}

int main(void)

{

ADC_init();          // initialize ADC

DDRB  = 0xFF;        // set all port b ports as output

//int val = 0;

step = 0;

TCCR0A |= (1 << WGM01);           // CTC mode

TCCR0B |= (1 << CS01) | (1 << CS00);   // Prescalar of 64

TIMSK0 |= (1 << OCIE0A);               // turn on overflow interrupt

OCR0A = 200;                      // set initial match value

sei();                   // turn on interrupt

while (1)

{

}

}

- SCHEMATICS

Task1:

Task2:

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

N/A

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task1:

Task2:

- VIDEO LINKS OF EACH DEMO

Task 1: https://www.youtube.com/watch?v=_ncwYZNwnsk

Task 2: https://www.youtube.com/watch?v=FJHlcehPWT0

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Brian Lopez