Brian Lopez

Nathan Hanuscin

CPE301 – SPRING 2018

# Design Assignment 4

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 1 | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 2. | INITIAL CODE OF TASK 1/A |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 4. | SCHEMATICS |  |  |
| 5. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 5. | SCREENSHOT OF EACH DEMO |  |  |
| 6. | VIDEO LINKS OF EACH DEMO |  |  |
| 7. | GOOGLECODE LINK OF THE DA |  |  |

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Disclaimer: My partner for this assignment is Nathan Hanuscin

- Atmega328P x 2

- FTDI x 2

- NRF24l01 x 2

- One LM34

- Jumper cables

- CODE FOR RECEIVER

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <string.h>

#include <stdbool.h>

#include <stdio.h>

#include <avr/interrupt.h>

#include "nrf24l01.h"

volatile bool rf_interrupt = false;

// Set up all of the SPI ports with the nRF struct and turn on int0 interrupt

nRF24L01 *setup_rf(void){

nRF24L01 *rf = nRF24L01_init();

rf->ss.port = &PORTB;

rf->ss.pin = PB2;

rf->ce.port = &PORTB;

rf->ce.pin = PB1;

rf->sck.port = &PORTB;

rf->sck.pin = PB5;

rf->mosi.port = &PORTB;

rf->mosi.pin = PB3;

rf->miso.port = &PORTB;

rf->miso.pin = PB4;

EICRA |= _BV(ISC01);

EIMSK |= _BV(INT0);       // turn on int0 interrupt

nRF24L01_begin(rf);

return rf;

}

void spi_init() {

DDRB &= ~((1<<2)|(1<<3)|(1<<5));   //SCK, MOSI and SS as inputs

DDRB |= (1<<4);     // MISO as output

SPCR &= !(1<<MSTR);    // set as slave

SPCR |= (1<<SPR0)|(1<<SPR1);   // divide clock by 128

SPCR |= (1<<SPE);        // enable SPI

}

void init_uart(){

// setting the baud rate  based on FCPU and baudrate

UBRR0H =0x00;

UBRR0L =0x0C;

// enabling TX & RX

UCSR0B = (1<<RXEN0)|(1<<TXEN0);              // enable receive and transmit

UCSR0A = (1<<UDRE0)|(1<<U2X0);

UCSR0C =  (1 << UCSZ01) | (1 << UCSZ00);    // Set frame: 8data, 1 stop

}

void ADC_init() {

ADMUX = 0;                 // read from port ADC0

ADMUX |= (1<<REFS0);       // use Vcc for reference

ADCSRA |= (1<<ADPS2) | (1<<ADPS1); // prescalar of 64

ADCSRA |= (1<<ADEN);       // enable ADC

ADCSRB = 0;                // free running mode

}

void USART_Transmit( char *data)

{

while((*data != '\0')) {    // transmits all chars but null

while(!(UCSR0A & (1<<UDRE0)));  // waits for transmit flag to clear

UDR0 = *data;           // transmit next char

data++;                 // move to next char

}

}

unsigned int readADC()

{

ADMUX &= ~(1<<ADLAR);              // clear the adc value

unsigned int val = 0;

ADCSRA |= (1 << ADSC);             // start adc

while(ADCSRA & (1<<ADSC));         // wait until adc is done

val = ADC;

val = val * 0.427;               // doing (5 * 100 * adc) / 1024, just simplified

return val;

}

// nRF24L01 interrupt

ISR(INT0_vect) {

rf_interrupt = true;    // turn on variable for while loop

EIFR |= (INTF0);   // reset interrupt flag

}

int main(void)

{

init_uart();         // set UART variables

ADC_init();          // set ADC variables

_delay_ms(150);

USART_Transmit("Started!\r\n");

uint8_t address[5] = {0x01, 0x01, 0x01, 0x01, 0x01 };   // address for nRF

sei();               // turn on global interrupts

nRF24L01 *rf = setup_rf();      // initialze and setup nRF struct

nRF24L01_listen(rf, 0, address);

uint8_t addr[5];

nRF24L01_read_register(rf, 0x00, addr, 1);

while (1)

{

if (rf_interrupt)

{

rf_interrupt = false;

while (nRF24L01_data_received(rf)) {

nRF24L01Message msg;

nRF24L01_read_received_data(rf, &msg);   // gets data that was sent

USART_Transmit((char *)msg.data);     // transmit the temperature value sent

USART_Transmit("\r\n");               // transmit a line feed

}

nRF24L01_listen(rf, 0, address);

}

}

return 0;

}

- CODE FOR TRANSMITTER

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <string.h>

#include <stdio.h>

#include "nrf24l01.h"

#define UBRR_9600 51 // for 8Mhz with .2% error

#define F_CPU 8000000UL

#include <util/delay.h>

void spi_init(void);

void setup_timer(void);

nRF24L01 *setup_rf(void);

void adc_init(void);

void read_adc(void);

void USART_init( unsigned int ubrr );

void USART_tx_string( char *data );

volatile bool rf_interrupt = false;

volatile bool send_message = false;

volatile unsigned int adc_temp;

char outs[20];

int main(void)

{

uint8_t to_address[5] = { 0x01, 0x01, 0x01, 0x01, 0x01 };

spi_init();

USART_init(UBRR_9600); 					//Initialize the USART (RS232 interface)

USART_tx_string("Connected!\r\n"); 	      		//Display connected

_delay_ms(125); 					//wait a bit

sei();

nRF24L01 *rf = setup_rf();

adc_init();                    // start ADC

setup_timer();                 // setup timer for CTC every 1 second

while (true)

{

if (rf_interrupt)

{

rf_interrupt = false;

int success = nRF24L01_transmit_success(rf);

if (success != 0)

nRF24L01_flush_transmit_message(rf);

}

if (send_message)

{

read_adc();           // read ADC value

send_message = false;

nRF24L01Message msg;

snprintf(outs,sizeof(outs),"%3d\r\n", adc_temp);

USART_tx_string(outs);      // display temperature read

memcpy(msg.data, outs, 3);

msg.length = strlen((char *)msg.data) + 1;

nRF24L01_transmit(rf, to_address, &msg);  // transmit the temperature value

}

}

return 0;

}

void adc_init(void)

{

/** Setup and enable ADC **/

ADMUX = 0;				//select ADC0 Pin as input

ADMUX = (0<<REFS1)| 		//Reference Selection Bits

(1<<REFS0)| 				//AVcc - external cap at AREF

(1<<ADLAR); 				//ADC right Adjust Result

ADCSRA = (1<<ADEN)| 			//ADC ENable

(1<<ADSC)| 					//ADC Start Conversion

(1<<ADATE)| 				//ADC Auto Trigger Enable

(0<<ADIF)| 					//ADC Interrupt Flag

(0<<ADIE)| 					//ADC Interrupt Enable

(1<<ADPS2)| 				//ADC Prescaler of 64

(1<<ADPS1)|

(0<<ADPS0);

ADCSRB = 0;

}

nRF24L01 *setup_rf(void)

{

nRF24L01 *rf = nRF24L01_init();

rf->ss.port = &PORTB;

rf->ss.pin = PB2;

rf->ce.port = &PORTB;

rf->ce.pin = PB1;

rf->sck.port = &PORTB;

rf->sck.pin = PB5;

rf->mosi.port = &PORTB;

rf->mosi.pin = PB3;

rf->miso.port = &PORTB;

rf->miso.pin = PB4;

// interrupt on falling edge of INT0 (PD2)

EICRA |= _BV(ISC01);

EIMSK |= _BV(INT0);

nRF24L01_begin(rf);

return rf;

}

void read_adc(void)

{

adc_temp = 0;						//initalize temp to 0

ADCSRA |= (1<<ADSC);					//start the conversion

while((ADCSRA & (1<<ADIF)) == 0);

{

//wait for conversion to finish

}

adc_temp = ADCH;					//get temp value

}

void spi_init(void)

{

DDRB |= (1<<2)|(1<<3)|(1<<5); 		// SCK, MOSI and SS as outputs

DDRB &= ~(1<<4); 			// MISO as input

SPCR |= (1<<MSTR); 			// Set as Master

SPCR |= (1<<SPR0)|(1<<SPR1); 		// divided clock by 128

SPCR |= (1<<SPE); 			// Enable SPI

}

// setup timer to trigger interrupt every second when at 8MHz

void setup_timer(void)

{

TCCR1B |= _BV(WGM12);

TIMSK1 |= _BV(OCIE1A);

OCR1A = 31250;

TCCR1B |= _BV(CS12);

}

/* INIT USART (RS-232) */

void USART_init( unsigned int ubrr )

{

UBRR0H = (unsigned char)(ubrr>>8);		//set baud rate

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0) | (1 <<RXEN0); 		// Enable receiver, transmitter

UCSR0C = (1 << UCSZ00) | (1 << UCSZ01); 	//asynchronous 8-bit data 1 stop bit

}

/* SEND A STRING TO THE RS-232*/

void USART_tx_string( char *data )

{

while ((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)))

{

//wait for the transmit buffer to empty

}

UDR0 = *data;				//put the data into the empty buffer, which sends the data

_delay_ms(125); 			// wait a bit

data++;

}

}

// each one second interrupt

ISR(TIMER1_COMPA_vect)

{

send_message = true;

TIFR1 |= (1<<OCF1A);

}

// nRF24L01 interrupt

ISR(INT0_vect)

{

rf_interrupt = true;

EIFR |= (1<<INTF0);

}

- SCHEMATICS

For Receiver:

For Transmitter:

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

N/A

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Receiver:

Transmitter:

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=BUFPaYAD4H8

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Brian Lopez